- Just sharing the way I modified the CDS-65 to also be a Service Dock

- [23:54]

It can still be used as a dock when not connected to the USB to TTL adapter

- [23:59]

Using this pinout, pins can also be soldered to a female end to a cst-75 charger. Only GND, RX and TX are necessary. The charger can also be plugged it for power

05 December 2024

- @GoZiLlA98

Just sharing the way I modified the CDS-65 to also be a Service Dock

- is that VPPFLASH?

- @Clara Thrace

is that VPPFLASH?

GoZiLlA98 — 05/12/2024 08:07

No. I broke the VCC pad and jumped it with a wire

Clara Thrace — 05/12/2024 08:07

ah

- [08:08]

I see, you used TXD on VPPFLASH pin too

GoZiLlA98 — 05/12/2024 08:08

For fastport, all is needed is TXD connected with VPPFLASH

Clara Thrace — 05/12/2024 08:09

Yeah, technically should work

- [08:09]

as TXD is pulled up

GoZiLlA98 — 05/12/2024 08:10

Yes. And this will keep the dock working when it’s not in use as a service cable

- [08:11]

If it was tied to 5v, the phone will automatically boot to bootrom when connected

Clara Thrace — 05/12/2024 08:11

I'll probably go with the physical switch way (edited)

- Sure. I didn’t wanted to cut another hole into the dock

- [08:14]

Also, don’t glue the header where I’ve done it. It will interfere with the stand. Find another place( maybe in the back or more through the front)

### Clara Thrace — 05/12/2024 08:32

I'll probably only add the switch inside the dock

- [08:32]

I already have a nice adapter for connecting UART dongles to Type-A female sockets

- [08:33]

so I can just plug the dock into that via the normal USB cable

- @GoZiLlA98

If it was tied to 5v, the phone will automatically boot to bootrom when connected

### fixeria — 05/12/2024 12:47

perhaps this is specific to the phone you tested, I saw no reaction when I tried with K800

- [12:47]

it should be 3.3V, 5V worked in some cases but resulted in unstable operation

- @fixeria

it should be 3.3V, 5V worked in some cases but resulted in unstable operation

### Clara Thrace — 05/12/2024 12:51

TXD is a special case

- [12:51]

as it's not directly connected to power

- [12:51]

but pulled up

- [12:51]

but I would definitely not use that for this

### GoZiLlA98 — 05/12/2024 13:03

Don't know, but for me it worked perfectly. I've flashed/unlocked/replaced certificates to all my fastport sony ericsson phones